Aptimizer: Master Product Roadmap & Technical Remediation Plan

From Conceptual Planning to Legally Defensible Civil Engineering AI OS | September 2026

―――――――――――――――――――――――――――――――――――――――――――――――――――――――

# 1. Executive Context & Audit Summary

Aptimizer has established an extraordinary algorithmic foundation across Versions 1 to 3, boasting 324 built features including polygon setback generation, greedy and genetic layout packing, IS/NBC structural checks, CPM scheduling with congestion curves, and an in-app AI assistant (APT) backed by a 74-clause IS/NBC RAG registry. However, to transition from a conceptual feasibility tool to an enterprise-grade civil engineering operating system used by Tier-1 developers and contractors, Aptimizer must overcome critical gaps in CAD/BIM interoperability, structural realism, and statutory compliance.

# 2. Existing Features to Fix & Technical Remediation

1. Structural Column Biaxial Moments & High-Rise Foundations (backend/engineering.py)

WHAT is broken: Column sizing relies solely on axial capacity (IS 456 Cl. 39.3), severely undersizing columns subjected to seismic/wind overturning moments. Module 3 prescribes isolated footings even for tall towers.
HOW to fix it: Implement IS 456 Cl. 39.6 Bresler biaxial interaction curves [(Mux/Mux1)^an + (Muy/Muy1)^an <= 1.0]. Introduce an automated Foundation Advisor selecting RCC Raft Foundations (IS 2950) when contact pressure exceeds soil SBC, and Bored Cast-in-Situ Piles (IS 2911) for buildings >45m or on soft soil.
WHY: Structural consultants will immediately reject plans without biaxial moment and realistic foundation checks.

2. Procedural Floor Plan Hierarchy & Core Pinning (backend/floorplan/)

WHAT is broken: Room layouts are generated by slicing the perimeter footprint inward, producing unbuildable plans without pinned vertical circulation and MEP shafts.
HOW to fix it: Invert the hierarchy. Pin the central structural core first (two 1.5m fire stairs per NBC Part 4, passenger/fire lifts, and 4 MEP risers). Grow residential wings outward from the frozen core.
WHY: High-rise structural stability and MEP efficiency require continuous vertical riser alignment from foundation to terrace.

3. Statutory Area Accounting & RERA Carpet Schedule (backend/engine.py)

WHAT is broken: Relies on legacy Super Built-Up area heuristics. Under the Indian RERA Act 2016, selling apartments on super built-up is illegal.
HOW to fix it: Implement statutory Section 2(k) RERA Carpet Area (Internal Usable Area + Internal Partition Walls, strictly excluding exterior walls, service shafts, and balconies). Provide certified RERA schedules.
WHY: Indian real estate developers cannot legally launch or finance projects without certified RERA carpet figures.

4. BOQ Substructure vs Superstructure Split & CPWD DSR Mapping (backend/takeoff.py)

WHAT is broken: Concrete, steel, and earthwork are lumped as a project-wide uniform rate without distinguishing between foundation and typical floors.
HOW to fix it: Split BOQ into Package A (Substructure: deep excavation, shoring, raft, waterproofing) and Package B (Superstructure: repetitive slabs, columns, finishes). Map items to 5-digit CPWD Delhi Schedule of Rates (DSR) codes.
WHY: Banks and contractors bid substructure and superstructure separately. DSR mapping enables government/tender adoption.

5. Construction Programme Climate Windows & Tower Crane Cycles (backend/schedule.py)

WHAT is broken: Schedule assumes uniform year-round productivity and infinite material hoist speed.
HOW to fix it: Add regional Monsoon Calendar Blackouts (June–September) for excavation/external plastering. Constrain floor cycles by Tower Crane hook cycle times [Cycle = max(Striking, Tonnage/Crane Hoist Rate)].
WHY: Any contractor will reject a schedule showing basement excavation in Mumbai or Bengaluru during peak July monsoons.

6. Internal Road Geometry & Fire Tender Swept-Path (backend/siteplan/reserve.py)

WHAT is broken: Road network junctions are sharp 90-degree polygon vertices lacking fire tender clearance.
HOW to fix it: Apply a fillet curve with minimum inside turning radius Rin >= 9.0m (12.0m outside) per NBC Part 4.
WHY: Fire NOC is the single most common cause of municipal site plan rejection.

# 3. What to Do Next: Multi-Phase Product Roadmap

| Phase & Timeline | Key Deliverables | Core Technology | Strategic Impact |
| --- | --- | --- | --- |
| Phase 1: Interoperability Bridge (Weeks 1–3) | • 1-Click Layered AutoCAD (.dxf) • OpenBIM IFC4 3D Model Export • ETABS (.s2k) / STAAD (.std) Export | ezdxf, ifcopenshell, CSI / Bentley parsers | Eliminates CAD silo; allows architects & structural engineers to adopt Aptimizer immediately. |
| Phase 2: Civil & Municipal Rigor (Weeks 4–7) | • 3D Earthwork Cut-and-Fill Balancer • City DCR Engines (Mumbai, BLR, HYD) • PreDCR Automated Sanction Export | TIN / Grid Prismoidal, Local Master Plans, PreDCR | Saves millions in earthwork hauling; unlocks automated municipal sanction approvals. |
| Phase 3: Headless CAD Sheets (Weeks 8–12) | • Headless FreeCAD TechDraw Worker • Automated A1/A0 Sanction Drawing Sheets • 3D Spatial Redlining & Approval Gates | Headless FreeCAD, Celery/Redis, Three.js | Replaces manual drafting of submission sheets; enables multi-disciplinary enterprise sign-offs. |
| Phase 4: Autonomous Civil OS (X) (Weeks 13–18) | • Multi-Agent Engineering Swarm • Live Commodity Price Tender Bundler • Drone Survey vs CPM As-Built Verification | Agent Swarms, Wholesale Indices, Photogrammetry | Delivers the complete Aptimizer X vision: autonomous engineering and real-time site monitoring. |

# 4. The Technology Architecture: FreeCAD vs. Pure Python

Why FreeCAD cannot replace the interactive web application, but serves as an exceptional headless worker:

1. Interactive Client (React 19 + Three.js): FreeCAD is a C++/Qt desktop app. It cannot deliver 60fps in-browser interactive editing. The web client must remain Three.js + Canvas2D.
2. Fast 2D CAD (ezdxf): FreeCAD takes 2-3 seconds to boot; ezdxf exports clean layered AutoCAD files in <40ms inside FastAPI.
3. OpenBIM (ifcopenshell): ifcopenshell generates certified IFC4 files directly in Python without C++ dependencies.
4. Headless FreeCAD Worker: Headless FreeCAD is reserved for asynchronous Celery workers to run TechDraw projection sheets (A1/A0 drawing cuts) and 3D B-Rep Boolean terrain earthworks.